AM 211 FRICTION EXPERIMENT

Laboratory Report

Name: ______________________    Matric No: ______________    Date: ____________

# SECTION C

# 1. Theory

Friction is the force that resists motion between two surfaces in contact. It acts along the surfaces, opposite to the direction of motion. Its size is given by

F = μN

where μ is the coefficient of friction (it has no units and depends on the two surfaces) and N is the normal reaction. Friction does not depend on the area of contact.

Level plane (Part A1): When the tray moves at a slow, constant velocity, it is not accelerating, so the net force is zero. The pulling force P equals the friction force, and the normal reaction equals the weight, N = W. So P = μW, which gives μ = P/W. A graph of P against W is a straight line whose slope is μ.

Angle of friction (Part A2): On a tilted plane, the weight has two components: W sin θ along the plane (trying to slide the tray down) and W cos θ into the plane (the normal reaction). At the critical angle the tray just starts to slide, so W sin θ = μW cos θ. This gives tan θ = μ, and θ is called the angle of friction (or angle of repose).

Inclined plane (Part B): The tray is pulled parallel to the plane at constant velocity.

- Up the plane, friction and W sin θ both act down the slope, so P = W(sin θ + μ cos θ).

- Down the plane, friction acts up the slope and W sin θ helps the motion, so P = W(μ cos θ − sin θ).

Rolling friction: With ball-bearing wheels, sliding is replaced by rolling, which has much less resistance. With no friction the force needed would be P = W sin θ. The actual force is P_A = P + F, where F is the rolling resistance, so F = P_A − W sin θ.

# 2. Apparatus

Mild steel plane (adjustable angle), slider trays with different liners, cord, pulley at the end of the plane, hanger and slotted masses, ball-bearing tray (three wheels), laboratory scale, protractor or angle scale, and a ruler.

[Draw a neat, labelled sketch here: inclined plane, tray, cord over the pulley, hanger with masses, angle θ marked.]

# 3. Results

Table 1: Level plane (Part A1). One table per tray material.

| Load added (N) | Total W (N) | Hanger + masses P (N) | P/W |
| --- | --- | --- | --- |

Table 2: Angle of friction (Part A2)

| Tray / liner | Critical angle θ | tan θ | μ from Table 1 |
| --- | --- | --- | --- |

Table 3: Inclined plane (Part B)

| θ | W (N) | P exp. (up) | P theory (up) | P exp. (down) | P theory (down) |
| --- | --- | --- | --- | --- | --- |

Table 4: Rolling friction

| θ | W (N) | P = W sin θ (N) | P_A measured (N) | F = P_A − P (N) |
| --- | --- | --- | --- | --- |

# 4. Analysis of Results

One worked example for each calculation, using my own readings:

- μ from graph: μ = slope = ΔP/ΔW = [ ] ÷ [ ] = [ ]  (2 decimal places)

- μ from angle: μ = tan θ = tan [ ] = [ ]

- Theoretical P (up): P = W(sin θ + μ cos θ) = [ ] N

- Theoretical P (down): P = W(μ cos θ − sin θ) = [ ] N

- Rolling resistance: F = P_A − W sin θ = [ ] − [ ] = [ ] N

- Percentage difference: (experimental − theory) ÷ theory × 100 = [ ] %

# 5. Observations / Precautions Taken

Observations

- The friction force increased as the load increased.

- The force needed to pull the tray up the plane was larger than the force needed to pull it down.

- The ball-bearing tray needed much less force than the sliding trays.

- Different liner materials gave different values of μ.

Precautions

- The plane was levelled before Part A.

- The tray was moved at a slow, constant velocity, not accelerated.

- The cord was kept parallel to the plane.

- Readings were taken at eye level to avoid parallax error.

- The trays and masses were weighed on the scale, and the plane was kept clean and dry.

# 6. Graphs

- Graph of P against W for [material 1]

- Graph of P against W for [material 2] (one graph per tray, on the graph sheets provided)

# 7. Discussion

Sources of error

- It is hard to tell when the tray is moving at exactly constant velocity, so the added mass is slightly off.

- Friction in the pulley and stiffness of the cord.

- Parallax when reading the protractor, so θ is only accurate to about ±[ ]°.

- Dust or uneven surfaces changed μ from one run to another. This is why the manual says to expect some scatter.

- The cord may not have been perfectly parallel to the plane.

Agreement with theory. My μ from the graph was [ ] and from tan θ was [ ]. They [agree closely / differ by [ ] %], which is [within / outside] the expected experimental error. My experimental values of P were [close to / different from] the theoretical values by about [ ] %. The differences can be explained by the errors above.

Rolling friction. The rolling resistance was much smaller than sliding friction, because rolling contact has far less resistance than sliding contact.

# 8. Conclusion

The coefficient of friction between the mild steel plane and the trays was found to be [ ] for [material]. tan θ agreed with μ to within [ ] %, and the experimental forces on the incline agreed reasonably with P = W(sin θ ± μ cos θ). Rolling friction was much smaller than sliding friction. Practical applications include the design of brakes, tyres and conveyors, choosing safe ramp angles, and using bearings to reduce friction in machines.

# SECTION B: QUESTIONS

(Answer only the questions assigned to you. Questions 1, 2, 10, 11, 12 and 14 are personal, so fill them in yourself.)

3. Comment on the experimental procedure. Adding load in steps and plotting a graph gives an average μ instead of relying on a single reading, which improves accuracy. Using several trays on the same plane saves time.

4. Most difficult aspect. Keeping the tray at a truly constant velocity. This can be reduced by adding mass in very small steps and giving the tray a light tap to start it.

5. Other method. A spring balance can pull the tray directly and read the friction force. A tilting plane on its own can also be used to find tan θ.

6. Parameters. Weight of the tray and load, hanger mass, angle of inclination, pulling force P, and the liner material.

7. Measuring instrument. The protractor (angle scale) measures the angle θ of the plane.

8. Practical relevance. Brakes, tyres and road grip, conveyor belts, ramps, and bearings.

9. Section of course. Statics (friction) in Applied Mechanics.

13. What interested me most. The ball-bearing tray moved with far less force than the sliding trays.

15. References. [List only the books you actually used, e.g. AM 211 lecture notes and your mechanics textbook.]